InFuse-origin-review-SIG

Reviewer 1

Description

This paper introduces a novel hybrid manufacturing approach that integrates additive manufacturing with material injection. While traditional methods require pre-printing internal airflow channels for subsequent injection, this new technique eliminates that necessity by interleaving the printing and injection phases. To optimize efficiency, the authors present a graph-based search algorithm designed to determine the ideal sequence planning and minimize nozzle travel time.

Clarity of Exposition

Line 324 (Eq. 1): Please clarify the inclusion of v_{travel}. If the velocity is constant, it does not influence the optimization of the sequence; therefore, its necessity in the equation is unclear.

Line 389: The construction of Graph G appears to be a core contribution of this work. Moving the majority of this detail to the supplementary material hinders the readability and flow of the methodology. I recommend moving the primary construction logic into the main text.

Line 470: The section on graph simplification is currently quite abstract and difficult to follow. The authors should provide illustrative figures to visualize the simplification steps.

Line 505: While addressing interlayer gaps is important, this post-processing step distracts from the primary focus of the paper. I suggest relocating this section to the supplementary material to maintain the narrative's momentum.

Quality of References

Should cite these two papers:

Yongxue Chen, Tao Liu, Yuming Huang, Weiming Wang, Tianyu Zhang, Kun Qian, Zikang Shi, and Charlie C.L. Wang, "Can any model be fabricated? Inverse operation based planning for hybrid additive-subtractive manufacturing", ACM Transactions on Graphics (SIGGRAPH Asia 2025)

Ziqi Wang, Florian Kennel-Maushart, Yijiang Huang, Bernhard Thomaszewski, and Stelian Coros. "A Temporal Coherent Topology Optimization Approach for Assembly Planning of Bespoke Frame Structures." ACM Transactions on Graphics (SIGGRAPH 2023)

Technical Correctness and Reproducibility

Good to me.

Validation

The justification for graph simplification remains unclear. Reducing the node count is not a sufficient proxy for improved search performance. It is essential to provide a direct statistical comparison of search times between the unsimplified and simplified graphs. Can Table 3 be interpreted as this comparison?

The current results show a reduction in search time (e.g., 30 seconds), which is negligible compared to a total manufacturing time of several hours. To demonstrate the true value of this approach, the authors should instead quantify the reduction in actual printing/travel time on the hardware.

The paper lacks a comparison with a conventional layer-by-layer approach. Is a layer-by-layer sequence is always feasible? A quantitative comparison is needed to prove that the proposed hybrid sequence offers a significant advantage over this standard baseline.

Ethics

Good to me

Explanation of Rating

The paper introduces an inspiring new direction by integrating material injection with 3D printing. The physical hardware setup and prototyping are impressively executed, and I have no doubt this technology will catalyze future research. I still have the following two questions:

It remains unclear where the primary technical contribution lies within the software framework. While formulating the process as a mathematical problem is a commendable first step, the manuscript fails to demonstrate why this specific problem is computationally 'hard.' A critical omission is a comparison against a conventional layer-by-layer approach; the authors should clarify if—and why—such a baseline would fail or be significantly less efficient.

Furthermore, since the constraints and objectives in Equation 1 appear to be linear or quadratic, a Mixed-Integer Linear Programming (MILP) formulation might offer a more robust solution. Alternatively, a learning-based approach could be explored for sequence planning. Above all, I urge the authors to quantify the actual impact of sequence optimization on total fabrication time before justifying the need for a specialized new algorithm.

Explanation of Conference vs Journal Recommendation

The validation lacks important aspects and I recommand only conference acceptation.

Reviewer 2

Description

The paper presents a fabrication workflow that combines 3D printing and material injection. The main contribution is a planning algorithm and a customized fabrication system.

Clarity of Exposition

While the main ideas of the paper is clearly presented and overall the writing is well-structured with didactic illustration and result figures, important algorithmic details are missing from the main text and deferred to the appendix (or in fact supplementary material in this case since it's not included in the document whose page number matters for the conference vs journal track). This makes it difficult to understand the actual contributions of the work.

Quality of References

Good. The related work section discuss relevant prior work and divide them into three categories, which set the context of the main problem well.

Technical Correctness and Reproducibility

The method of the paper chains together multiple techniques including DAG construction and beam search algorithm. However, there are several unclear aspects:

1.In many of the convincing examples in the result section, including pneumatic structures and sacrificial mold, multi-cavities are needed. However the algorithm presented focus on single cavity and in this case even the supplementary document does not provide sufficient details. For example the definition of M + and M++ is unclear.

2.It's unclear why graph simplification is needed. Providing complexity analysis for the algorithm and an ablation isolating the effect of the simplification step would help.

The steps in the method seem highly dependent on the printing contours which is given as an input.

3.How does the printing width or nozzle dimeter affect the fabrication process? The supplementary video shows that the bunny ears are almost entirely 3D printed rather than in-situ injection. What does the current algorithm do when a single cavity model becomes multi-cavity due to thin features and printing thickness. Moreover, the paper motivates hybrid fabrication as a way to improve isotropy and structural performance, yet in regions with thin features which are typically the weak points the hybrid strategy appears not to be applied. So it's unclear if there are actual structural improvement when the method is applied on real models than just test samples.

Validation

The method is validated by a large set of fabricated examples. The supplementary video also showcases the fabrication process well. See the issue raised in the last point of the section above.

Ethics

N/A

Explanation of Rating

My main concern is that the paper is difficult to evaluate from a technical standpoint because many essential algorithmic details are deferred to the supplementary document rather than presented in the main paper. Almost all core algorithmic components are missing including constraint definition, DAG construction, graph simplification algorithm, beam search algorithm. As a result, the technical scope and contributions of the work remains unclear. It's also difficult to place the paper primarily as a planning algorithm paper or a fabrication system paper.

The supplementary document is 10 pages, which would be difficult to be fully incorporate into the main text during the revision period. Therefore I suggest the authors to resubmit after a major revision.

Explanation of Conference vs Journal Recommendation

The current format of the paper is not suitable for either; see reasons above.

Reviewer 3

Description

This paper introduces a hybrid manufacturing framework that integrates 3D printing and material injection within a single automated pipeline. A DAG-based sequence planning algorithm for dependency-aware process ordering, coupled with an interlayer gap treatment scheme and a modular software–hardware platform featuring a custom user interface. The approach is validated through several physical examples, demonstrating reduced fabrication time and improved material isotropy.

Clarity of Exposition

The paper presents an interesting direction in integrating printing and injection processes into one closed-loop framework.

However, the technical contributions of the paper remain unclear and somewhat incremental. The three contributions claimed in the introduction seem largely engineering-level system integration rather than a computational innovation.

The core sequence planning algorithm, as described, appears to be a straightforward application of DAG-based dependency graph traversal and beam search, showing less conceptual difference from existing work on sequence or toolpath planning in 3D printing/hybrid manufacturing (e.g., ACAP [Zhong et al. 2023a] and VASCO [Zhong et al. 2023b]). It is not clearly explained what is new in the computational part, how this approach differs from prior graph-based process planning methods, or why it is uniquely suited to material injection scenarios.

From a fabrication standpoint, bonding strength and material compatibility need more discussion. In particular, the interfacial stiffness and adhesion between silicone and 3D-printed thermoplastics are known challenges but are not addressed in detail here. A similar idea of machine modification was reported 10 years ago: Ulticast HD - details can be found at https://www.youtube.com/watch?v=vNM3bI0jAOg.

Quality of References

The integration of silicon injection with 3D printing has been reported, and references should be included.

For example, Hybrid Formative‐Additive Manufacturing, Adv. Mat. 2025

Bonding between silicones and thermoplastics using 3D printed mechanical interlocking, Materials & Design, 2020.

Technical Correctness and Reproducibility

Authors have provided sufficient information on the implemtation and I expect the work can be reproduced without problem.

Validation

Although the validation is comprehensive and includes multiple physical fabrications, it does not convincingly demonstrate novel capabilities. Authors give strong claims about handling arbitrary geometries, the demonstrated examples (e.g., Bunny, Pipe, Hand) are still relatively simple. The robustness of the algorithm/fabrication setup can be better demonstrated in the model as illustrated in [Yan et al. 2019].

The runtime (around two hours for complex cases) also raises concerns about scalability. The analysis of computational bottlenecks or drawbacks of existing optimization strategies is expected to be clearly tested and discussed.

Ethics

N.A.

Explanation of Rating

The paper presents a well-engineered hybrid manufacturing platform, but the computational originality and algorithmic contribution are limited.

Explanation of Conference vs Journal Recommendation

The paper proposed considerable technical details and thorough validation, with many details included in the supplementary documents, which can be considered as a journal track.

Reviewer 4

Description

This paper proposes graph-based algorithm that enables in-situ injection while 3D printing process. This algorithm takes various fabrication constraints (for example, nozzle collision with printed objects, overflow of injected material) into consideration.

This algorithm can also be used to print customized mold with dissolvable material as shown in Fig. 16.

Clarity of Exposition

This paper is dual-track submission and satisfies the length guidelines.

While the content is well-organized and the manuscript remains focused, the Appendix is too extensive (10 pages), and I think it might be worth considering whether all of the detailed exposition in Appendix is essential or can be shortened.

Because there are very detailed Appendix, the exposition is very clear for me.

Quality of References

This paper contains sufficiently many references, I think.

Technical Correctness and Reproducibility

The problem is converted as an optimization problem over graph, and the algorithm looks correct.

And, the correctness is also validated in various aspects (calculation time measurements, actual fabrication, user studies).

As I mentioned above, there are very detailed Appendix and I think it is very straightforward to reproduce the proposed method. One minor concern is the difficulty of reproducing the physical equipment, but this is very minor thing.

Validation

I already mentioned briefly, this paper validates their methods from various aspects, such as calculation time measurements, actual fabrication, user studies.

Based on the reported results, their methods are superior to the existing methods.

Also, the fabricated objects are visually good (plus, better than traditional printing-then-injection and manually interleaved printing-injection).

I'm convinced by these various validations and evaluations.

Ethics

None

Explanation of Rating

The problem is formulated as graph-based problem and this paper proposes clear solution for that (with very detailed exposition).

In addition to that, there are sufficient validation and evaluation from various aspects.

I think this work is well-validated and good enough for SIGGRAPH.

One concern is, the Appendix is too long and perhaps the authors need to shorten it and prioritize the most essential contents during the revision.

Explanation of Conference vs Journal Recommendation

As I mentioned above, this work is well-validated and suited for journal acceptance (if accepted), I think.