We thank all reviewers for their constructive feedback. We appreciate that the core idea (R1, R2), its potential to inspire future research (R1), and the reproducibility of our work (R3, R4) have been recognized. This rebuttal addresses the major concerns; minor issues will be corrected in the revised manuscript.

**Move algorithmic details from supplementary to main paper (R1,R2)**

Due to the dual-track 7-page limit, we placed some algorithmic details in the supplementary.

In the revision, we will condense background and non-essential sections to move core technical details (constraint definition, DAG construction, graph simplification, beam search) into the main text, and relocate interlayer gap treatment to the supplementary material.

If accepted for the journal-track (as suggested by R3 and R4), the absence of strict page limits allows us to present all core technical details in a fully self-contained manner without excessive condensation.

**Clarification of technical contribution (R2,R3)**

This paper is not a system integration work. Instead, it is a computational fabrication and planning paper, with the hybrid platform serving as validation. To reiterate, our key technical contributions are:

1) This paper proposes a novel problem of constrained sequence planning for hybrid injection-printing, which we formulate mathematically to model the coupled constraints of the two processes. In this formulation, printed surface contours and injectable cavity volumes must be planned together, leading to a new surface–volume co-segmentation problem.

2) We further formulate injection feasibility as geometric constraints and couple them with printing constraints in a unified representation. The coupled constraints are encoded into a dependency DAG for sequence planning, supported by a graph simplification algorithm and an interlayer gap treatment to improve computational efficiency and manufacturability.

These contributions distinguish our work from existing graph-based planning methods for printing and machining (e.g., ACAP/VASCO).

**Why this specific problem is computationally 'hard.' (R1)**

The problem is computationally challenging due to its combinatorial nature: valid sequences must be generated under tightly coupled constraints (printing order, collision avoidance, injection feasibility). These constraints cannot be decomposed into independent subproblems and jointly restrict surface ordering and cavity filling, leading to a Precedence-Constrained Minimum Path Cover Problem (PC-MPC) over a large DAG—a known NP-hard scheduling problem whose search space grows quickly with model complexity.

**Comparison with layer-by-layer (R1)**

A direct comparison with a standard layer-by-layer method is not feasible, as it does not account for injection constraints. Within the InFuse framework, we implemented a layer-by-layer baseline by replacing beam search with a fixed layer-wise ordering.

Compared with this baseline, InFuse achieves higher manufacturing efficiency because the layer-by-layer causes long jumps between disconnected contours (substantial idle time), while InFuse groups contours to reduce jumps.

For example, in the Bunny and Pipe models shown in Fig.10, the layer-by-layer strategy produces idle-travel distances and long-jump counts of (3260 mm,29) and (11810 mm,133), with fabrication times of 184.1 min and 231.9 min. In contrast, InFuse reduces them to (149.1 mm,5) and (816.7 mm,17), with fabrication times of 171.2 min and 181.1 min.

**Provide illustrative figure for graph simplification (R1)**

An illustrative figure has been provided in Fig. 6 in Appendix E.

**Provide a direct statistical comparison of search times between the unsimplified and simplified graphs (R1)**

Table 3 does not compare simplified and unsimplified graphs. This ablation study is included in Appendix K, where idle travel distance is used to demonstrate the practical benefit of graph simplification.

**Clarify the inclusion of v_{travel} (R1)**

In Equation 1, v_{travel} is a constant that does not affect sequence optimization. We will remove it to simplify the formula.

**MILP and learning-based approach (R1)**

We agree that both are promising directions for future work.

**Why graph simplification is needed? (R2)**

Graph simplification removes redundant nodes in the dependency graph, reducing the number of candidate sequences explored during planning and improving search efficiency.

**More details on multi-cavities (R2)**

Our algorithm supports both single-cavity and multi-cavity models, which differ only slightly in graph construction, detailed in Appendix D.

M+ and M++ denote the non-injection and injection cavities of the multi-cavity model, respectively, as illustrated in Fig. 5 of the appendix.

**Thin-features related concerns (R2)**

Nozzle diameter defines the minimum injectable contour size, limiting injection of thin features (implementation details in Sec.4.1).

In our algorithm, a single-cavity model will not be split into multiple cavities; cavities are determined according to the nozzle diameter during the initialization of the connected cavity graph.

For thin-features that do not satisfy the nozzle constraint, the algorithm uses 3D-printed solid infill. Even so, injected regions in real structures still benefit from reduced anisotropy, while the hybrid strategy also improves overall manufacturing efficiency.

**Examples complexity and algorithm robustness (R3)**

We evaluated InFuse on the TPMS-channel model (Fig.1) [Yan et al. 2019], with a runtime of 553.9 s and 4 injection operations.

To further demonstrate robustness, we tested 50 randomly selected models from the Thingi10K dataset. All cases were successfully processed (average runtime ~10 minutes).

**Computational bottlenecks (R3)**

The main computational bottlenecks are beam search and interlayer gap treatment (Section 6.2). Beam search performance mainly depends on the DAG complexity, while interlayer gap treatment depends on the number of contours as Boolean operations are applied to each contour.

**Material compatibility (R3)**

This work focuses on hybrid injection-printing sequence planning, rather than detailed material interface analysis.

Feasibility is validated via physical fabrication and tensile tests, with material details provided in Appendix J. A systematic study of interfacial bonding is left for future work.

**Appendix is too extensive (R4)**

We will streamline the presentation in the revision, particularly for the user interface and user study sections.

**References (R1,R3)**

The suggested references will be added and discussed.